# Voice and Speech Capture

The Text to Speech and Speech to Text allows the system to feel alive and more interactive, allowing for visitors to feel comforted and intruders to feel more at an edge. The code is as follows:

# speech.py (renamed from audio_interface.py)

import speech_recognition as sr

import pyttsx3

import asyncio

import time

class STT_Engine:

"""Speech-to-Text using microphone input."""

def __init__(self):

self.recognizer = sr.Recognizer()

print("[STT] Engine initialized")

def listen_for_visitor(self, timeout=10):

with sr.Microphone() as source:

print("[STT] Listening for visitor speech...")

try:

audio = self.recognizer.listen(source, timeout=timeout)

text = self.recognizer.recognize_google(audio)

print(f"[STT] Heard: {text}")

return text

except sr.WaitTimeoutError:

print("[STT] Timeout — no speech detected.")

return ""

except sr.UnknownValueError:

print("[STT] Could not understand speech.")

return ""

except sr.RequestError as e:

print(f"[STT ERROR] {e}")

return ""

class TTS_Engine:

"""Text-to-Speech using pyttsx3 (offline)."""

def __init__(self):

self.engine = pyttsx3.init(driverName='espeak')

self.engine.setProperty("rate", 150)

print("[TTS] Engine initialized")

def speak(self, text):

print(f"[TTS] Speaking: {text}")

self.engine.say(text)

self.engine.runAndWait()

def play_loud_sound(self):

self.speak("ALARM ACTIVE! Security services notified.")

class VisitorInteraction:

"""

Handles the full visitor interaction sequence with OzBot:

- Wait 3 sec after motion

- Ask for reason (STT)

- Send transcript + snapshot to user

- Wait for accept/deny using callback

- Update display + TTS accordingly

"""

def __init__(self, display, bot, snapshot_path=None):

self.display = display

self.bot = bot

self.stt = STT_Engine()

self.tts = TTS_Engine()

self.snapshot_path = snapshot_path

self._future_response = None # Will be set when waiting for homeowner

async def interact_with_visitor(self):

# 1️⃣ Show welcome face for 3 seconds before starting STT

print("[Interaction] Visitor detected, showing welcome face...")

self.display.show_expression("HAPPY")

time.sleep(3)

# 2️⃣ Welcome prompt + STT capture

self.tts.speak("Welcome! Please tell me the reason for your visit.")

visitor_text = self.stt.listen_for_visitor(timeout=10)

if not visitor_text:

visitor_text = "No speech detected."

# 3️⃣ Show neutral while waiting for owner's decision

self.display.show_expression("NEUTRAL")

self.tts.speak("Processing your request...")

# 4️⃣ Send alert with snapshot to Telegram

self.bot.send_alert(self.snapshot_path, visitor_text)

# 5️⃣ Wait for homeowner response using callback

self._future_response = asyncio.get_event_loop().create_future()

def response_callback(action):

if not self._future_response.done():

self._future_response.set_result(action)

self.bot.register_callback(response_callback)

print("[Interaction] Waiting for homeowner decision...")

response = await self._future_response

# 6️⃣ Handle homeowner response

if response.lower() == "accept":

self.display.show_expression("HAPPY_CLOSED_EYES")

self.tts.speak("Access accepted. Please wait while we open the door.")

elif response.lower() == "deny":

self.display.show_expression("SAD")

self.tts.speak("Access denied. Sorry, you cannot enter.")

else:

self.display.show_expression("NEUTRAL")

self.tts.speak("Unknown response received.")